Aerial Fire Suppression Aircraft in Minnesota

Helicopters

|  | Type III | Type II | Type I |
| --- | --- | --- | --- |
| Water Dropping Capacity (Gals.) | 100 (Intermediate*-144) | 300 | 700 |
| Foam Capabilities | N/A | N/A | Injection System |
| Retardant Capabilities | Heli-Well | Heli-Well | Heli-Well |
| Tank / Bucket System | Bucket | Bucket / Belly Tank | Bucket / Belly Tank |
| Passenger Seats | 4-8 (Intermediate*-minimum 5) | 9-14 | N/A |
| Useful Load (Lbs.) | 1,200 | 2,500 | 5,000 |
| Type of Fuel | Jet A | Jet A | Jet A |
| Typical Fuel Cycle | 1.5 – 2.0 hours | 1.5 – 2.0 hours | 1.5 – 2.0 hours |
| Wind Limitations <500’ AGL | 30 mph – Gusts 15+ | 40 mph – Gusts 15+ | 40 mph – Gusts 15+ |
| Examples | 206L3, 206B3, 206L1, 500D Intermediate* - 206L4, 407, AS350 B2, AS350 B3 | 205A1++, 212 HP, 212, S-58T, UH-1H | S-64, S-61, Boeing 234 & 107, K-Max |

* Intermediate Helicopters are high performance Type III Helicopters, designated by MN DNR Forestry

Helicopter Operations Safety

Buffers around the drop zone must be maintained to provide a safe working environment for ground personnel.

These minimum standards should be followed:

- 50’ laterally for personnel & equipment

- 100’ in front of or behind personnel & equipment in grass/brush fuel types or canopied equipment in timber fuel types

- 1½ times the canopy height in front of or behind unprotected personnel in timber fuel types.