Aerial Fire Suppression Aircraft in Minnesota

Land-Based Tankers

|  | SEAT | Large Air Tankers | Very Large Air Tankers (VLAT) |
| --- | --- | --- | --- |
| Water Dropping Capacity (Gals) | 799 | Type I – > 3,000 Type II – 1,800-2,999 Type III – 800-1,799 | 10,000+ Gallons |
| Gel Capabilities | Load at Tanker Base | Load at Tanker Base | Load at Tanker Base |
| Retardant Capabilities | Load at Tanker Base | Load at Tanker Base | Load at Tanker Base |
| Drop Height | 60-100 ft. above canopy | 100-200’ above Canopy | 100-200’ above Canopy |
| Load Release | Two Clamshell Doors – Computer Controlled Constant Flow – Salvo, String, or Split Drops – Coverage Level 1-6 | Four Doors –Computer Controlled, Sequence or Salvo Coverage Level 1-6 | Four Doors – Computer Controlled, Sequence or Salvo Coverage Level 1-6 |
| Type of Fuel | Jet A | Jet A | Jet A |
| Fuel Burn/hour | 600 lbs/90 gal | 6700 lbs/1000 gal. | 18,090 lbs/2700 gal. |
| Min Runway Length | 3,000 gravel | 5,000 feet | 6,000 feet |
| Headwind Limitations | 40 mph | None | None |
| Crosswind | 20 knots | 22 knots | 22 knots |
| Fuel Cycle | 3 hours | 4 hours | 4 hours |
| Normal Cruise Speed | 140 knots | 350 knots | 380 knots |
| Max. Cruise Speed | 155 knots | 400 knots | 560 knots |
| Examples | Air Tractor 802 | Type I – C-130 MAFFS, BAe 146 Type II – P2V | DC-10 |

Tanker Operations Safety

Tanker operations are potentially dangerous. The entire load can be released in seconds. From drop heights of 100-200 feet, this force can have a devastating effect on tree crowns and shallow rooted tree species. It is essential that all line personnel stay clear of the drop zones.

Buffers around the drop zone must be maintained to provide a safe working environment for ground personnel.

These minimum standards should be followed:

- 200’ laterally for personnel & equipment

- 300’ in front of or behind canopied equipment

- 500’ in front of or behind unprotected personnel